CARBOT

System Setup & Operation Guide

Jetson Orin NX — Vision-Guided Robotic Arm

| Platform | NVIDIA Jetson Orin NX + JetPack 6 |
| --- | --- |

# 1. Hardware Overview

| Component | Details |
| --- | --- |
| SBC | NVIDIA Jetson Orin NX, Tanna TechBiz Eagle 201 carrier |
| OS / JetPack | Ubuntu 22.04 · JetPack 6 (L4T R36.5.0) |
| CUDA | CUDA 12.6, installed at /usr/local/cuda-12.6/ |
| Servos | 7× Dynamixel MX-28 on /dev/ttyUSB0 @ 222,222 baud |
| Servo IDs | 1–5 → absolute (arm joints); 6–7 → relative (pan/tilt) |
| Linear Actuator | GPIO pin IN3=35, IN4=37 (Jetson.GPIO) |
| Camera | USB camera, default /dev/video0 (index 0) |

# 2. Repository Structure

feature-carbot/

├── best.pt                  # Trained YOLO model weights

├── carbot.sh                # One-shot launcher (interactive menu)

├── requirements-vision.txt  # Python deps for vision stack

├── vision/                  # Vision module (Python package)

│   ├── camera.py            # OpenCV camera capture

│   ├── config.py            # Env-driven config (VisionConfig)

│   ├── detector.py          # YOLO / Roboflow detector

│   ├── annotate.py          # Draw bounding boxes on frames

│   ├── window_servo.py      # Main vision-to-motor loop

│   ├── motion_client.py     # TCP client → motion_server

│   ├── mjpeg_server.py      # Browser preview server

│   ├── runner.py            # Standalone inference logger

│   ├── server.py            # TCP server for latest detections

│   └── types.py             # ButtonDetection, BoundingBox types

└── carbot_main/

├── motion_server.py     # TCP motor control server (port 5000)

├── carbot_record.py     # Servo recorder / motion editor

├── actuator.py          # Linear actuator GPIO control

└── actions/             # Saved motion JSON files

# 3. Why Docker?

A standard pip install torch downloads a generic aarch64 PyTorch that cannot use the Jetson GPU — it requires JetPack-specific CUDA libraries pre-linked at compile time.

The dustynv/l4t-pytorch Docker image (built by NVIDIA partner Dustin Franklin) includes:

- Jetson-native PyTorch compiled against JetPack 6 / L4T R36

- All CUDA shared libraries pre-linked (libcublas, libcupti, libcusparseLt, etc.)

- TensorRT for model export and fast GPU inference

- OpenCV with CUDA acceleration compiled against the Jetson iGPU

🔴 IMPORTANT: Run the vision inference (YOLO) inside the Docker container and the motor server (motion_server.py) directly on the host. They communicate over TCP on 127.0.0.1:5000.

# 4. Docker: One-Time Setup

## 4.1  Verify the image is available

sudo docker images

# Expected:  dustynv/l4t-pytorch:r36.2.0

## 4.2  First-time run — install dependencies inside the container

Run the container once from the base image, install all dependencies, then commit it as carbot:ready. This only needs to be done once — all packages are permanently baked in.

# Step 1 — start an interactive container from the base image

sudo docker run \

--runtime nvidia \

--network host \

--privileged \

-it \

-v /home/jnx/Prajwal/feature-carbot:/workspace \

-v /dev:/dev \

dustynv/l4t-pytorch:r36.2.0 \

bash

# Docker assigns a random name (e.g. relaxed_nash) — note it down

# Step 2 — inside the container, install all deps

cd /workspace

pip3 install ultralytics --no-deps

pip3 install -r requirements-vision.txt --no-deps

# Fix NumPy version conflict (required once)

pip install "numpy==1.26.4" --break-system-packages --force-reinstall \

--index-url https://pypi.org/simple/

# Verify GPU works

python3 -c "import torch; print('GPU:', torch.cuda.is_available())"

# Expected: GPU: True

# Exit the container

exit

💡 TIP: Use --no-deps so pip does NOT replace the image's pre-built Jetson torch with a generic CPU-only version.

## 4.3  Find the container name and commit it

After exiting, find the container Docker created and commit it as a reusable image:

# See all containers and their auto-assigned names

sudo docker ps -a

# Example output:

# CONTAINER ID   IMAGE                         NAMES

# 42946a8b2229   dustynv/l4t-pytorch:r36.2.0   relaxed_nash   ← most recent

# 71f26cacb663   dustynv/l4t-pytorch:r36.2.0   amazing_lamarr

# Commit the most recently used container (relaxed_nash in this example)

sudo docker commit relaxed_nash carbot:ready

# Verify the image was saved

sudo docker images

# Should now show:  carbot:ready

# Confirmed saved image:

# IMAGE           ID             DISK USAGE

# carbot:ready    c26bc3481b99   21.9GB

💡 TIP: carbot:ready has all packages baked in. Even if you delete every container, spin up a new one from carbot:ready and everything is already installed — no pip commands needed.

## 4.4  Create the permanent named container from carbot:ready

Now create one named container from the saved image. You will reuse this container every session.

sudo docker run \

--runtime nvidia \

--network host \

--privileged \

-it \

--name carbot_vision \

-v /home/jnx/Prajwal/feature-carbot:/workspace \

-v /dev:/dev \

carbot:ready \

bash

🔴 IMPORTANT: Never use dustynv/l4t-pytorch again for daily use. Always start from carbot:ready or the carbot_vision container.

# 5. Starting the Container (Daily Use)

Now that packages are baked into the carbot:ready image, use this every session:

## Option A — Restart the existing named container (fastest)

# Start the stopped container and get a shell

sudo docker start -ai carbot_vision

# Or open an extra shell in an already-running container

sudo docker exec -it carbot_vision bash

## Option B — Spin up a fresh container from the saved image

Use this if you accidentally deleted the container or want a clean slate:

sudo docker run \

--runtime nvidia \

--network host \

--privileged \

-it \

--name carbot_vision \

-v /home/jnx/Prajwal/feature-carbot:/workspace \

-v /dev:/dev \

carbot:ready \

bash

All packages are already installed — no pip commands needed. Start working immediately.

| Docker flag | Purpose |
| --- | --- |
| --runtime nvidia | Activates NVIDIA container runtime so GPU is visible |
| --network host | Container shares host network; vision reaches 127.0.0.1:5000 |
| --privileged | Required for /dev/video0 (camera) and /dev/ttyUSB0 (servos) |
| -v /home/jnx/Prajwal/feature-carbot:/workspace | Mounts your code into the container |
| -v /dev:/dev | Passes all device nodes (camera, USB serial) into the container |
| --name carbot_vision | Names the container so you can restart without re-typing flags |

# 6. Full Startup Sequence

Open two terminals on the Jetson. Both must stay running simultaneously.

## Terminal 1 — Motor Control Server (HOST, not Docker)

cd /home/jnx/Prajwal/feature-carbot/carbot_main

python3 motion_server.py

What it does:

- Opens /dev/ttyUSB0 at 222,222 baud and pings all 7 servos

- Starts a TCP server on 0.0.0.0:5000

- Listens for JSON-line commands: servo_move, freeze, neutral, play, actuator, etc.

- Must stay running — all servo motion flows through here

Expected startup log:

Connected to Dynamixel bus.

LinearActuator initialised on pins 35/37.

Motion Server listening on 0.0.0.0:5000

## Terminal 2 — Vision + Tracking (inside Docker)

# Attach to the container

sudo docker start -ai carbot_vision

# Go to workspace and launch

cd /workspace

./carbot.sh

The launcher will present an interactive menu:

╔══════════════════════════════╗

║       CARBOT LAUNCHER        ║

╚══════════════════════════════╝

Select target button:

1. front_left_window

2. front_right_window

3. rear_left_window

4. rear_right_window

5. door_lock

6. window_lock

Enter number (1-6):

Or skip the menu and go directly to a button: ./carbot.sh front_left_window

What it does:

- Loads best.pt via Ultralytics YOLO on the Jetson GPU

- Opens camera index 0 (/dev/video0)

- Starts MJPEG preview at http://<jetson-ip>:8080/ — open on your laptop

- SEARCH phase: sweeps servo 6 (pan) and servo 7 (tilt) until a button is detected

- TRACK phase: sends servo_move commands to Terminal 1 to center the button in frame

🔴 IMPORTANT: carbot.sh only launches the vision side. motion_server.py in Terminal 1 must already be running before you start carbot.sh.

# 7. Tuning the Tracker

All tuning values are at the top of carbot.sh. Edit them once and they apply every run.

## Speed & responsiveness

| Variable | Default in carbot.sh | Effect |
| --- | --- | --- |
| TRACK_SPEED | 600 | Servo speed during tracking (0–1023). Higher = faster alignment. |
| SEARCH_SPEED | 400 | Servo speed during search sweep. |
| KP_X / KP_Y | 0.5 / 0.5 | Proportional gain. Higher = more aggressive correction. Reduce if oscillating. |
| MAX_DELTA | 200 | Max servo counts per tracking step. Caps how far it moves each tick. |
| DEADZONE | 20 | Pixel radius where no correction is sent. Increase to reduce jitter when centered. |
| CONFIDENCE | 0.5 | Minimum detection confidence (0–1). Raise to reduce false positives. |

## Direction inversion

If the arm moves away from the button instead of toward it, invert the relevant axis in carbot.sh:

INVERT_PAN=1    # flip horizontal direction

INVERT_TILT=1   # flip vertical direction

## Inference speed

# In carbot.sh or as env var — lower interval = faster reaction

export VISION_INFER_INTERVAL_SEC=0.2   # inference every 200ms

# 8. Environment Variable Reference

## Vision / Inference

| Variable | Default | Description |
| --- | --- | --- |
| VISION_RUNTIME | embedded | Set to yolo for local .pt file |
| VISION_MODEL_PATH | best.pt | Path to YOLO weights file |
| VISION_CONFIDENCE | 0.4 | Minimum detection confidence (0.0–1.0) |
| VISION_INFER_INTERVAL_SEC | 0.5 | Seconds between inference calls |
| VISION_LABEL_ALLOWLIST | (all) | Comma-separated class names to track, e.g. front_left_window |
| VISION_MOCK | 0 | Set 1 to disable inference (wiring tests) |

## Camera

| Variable | Default | Description |
| --- | --- | --- |
| CARBOT_VISION_CAMERAS | 0 | USB camera index |
| CARBOT_VISION_GSTREAMER | (unset) | Full GStreamer pipeline string (overrides index for CSI cams) |

## Pan/Tilt Servo Tuning

| Variable | Default | Description |
| --- | --- | --- |
| VISION_PAN_SERVO | 6 | Servo ID for horizontal pan |
| VISION_TILT_SERVO | 7 | Servo ID for vertical tilt |
| VISION_KP_X | 0.35 | Proportional gain for horizontal error |
| VISION_KP_Y | 0.35 | Proportional gain for vertical error |
| VISION_TRACK_SPEED | 180 | Servo speed during tracking (0–1023) |
| VISION_SEARCH_SPEED | 200 | Servo speed during search |
| VISION_SEARCH_STEP | 90 | Step size (raw counts) for search sweeps |
| VISION_MAX_DELTA | 140 | Max servo delta per tracking step |
| VISION_DEADZONE_PX | 28 | Pixel radius of center deadzone |
| VISION_INVERT_PAN | 0 | Set 1 if pan moves opposite to the error |
| VISION_INVERT_TILT | 0 | Set 1 if tilt moves opposite to the error |
| VISION_LOST_FRAMES | 10 | Frames without detection before returning to SEARCH |

## Preview Server

| Variable | Default | Description |
| --- | --- | --- |
| VISION_PREVIEW_HOST | 0.0.0.0 | MJPEG server bind address |
| VISION_PREVIEW_PORT | 8080 | MJPEG server port |
| VISION_PREVIEW_JPEG_QUALITY | 75 | JPEG quality (1–95); lower = faster stream |

# 9. Optional: TensorRT Export for Maximum FPS

Once GPU inference is working, export the model to TensorRT format for 3–5× faster inference:

# Inside Docker container

cd /workspace

yolo export model=best.pt format=engine workspace=4

# Outputs: best.engine  (takes ~5–10 minutes, one-time only)

# Run with the engine file

export VISION_MODEL_PATH=best.engine

./carbot.sh front_left_window

💡 TIP: TensorRT compilation only needs to be done once per model. The .engine file is saved next to best.pt and loads instantly on future runs.

# 10. Diagnostic Commands

# Check if GPU is active (run inside Docker)

python3 -c "import torch; print('GPU:', torch.cuda.is_available())"

# Check NumPy version (must be < 2.0 for PyTorch compatibility)

python3 -c "import numpy; print(numpy.__version__)"

# Check USB serial port

ls -la /dev/ttyUSB*

# Check camera device

ls -la /dev/video*

# Test camera opens (inside Docker)

python3 -c "import cv2; cap=cv2.VideoCapture(0); print('Camera OK:', cap.isOpened()); cap.release()"

# Check Docker container and image status

sudo docker ps -a

sudo docker images

# View container logs

sudo docker logs carbot_vision

# Open a second shell in the running container

sudo docker exec -it carbot_vision bash

# 11. Quick Reference Card

| Task | Command |
| --- | --- |
| Start motor server (host) | cd carbot_main && python3 motion_server.py |
| Attach to container | sudo docker start -ai carbot_vision |
| Launch tracker (menu) | cd /workspace && ./carbot.sh |
| Launch tracker (direct) | ./carbot.sh front_left_window |
| View MJPEG preview | http://<jetson-ip>:8080/ in browser |
| Stop tracker | Ctrl+C in Terminal 2 |
| Check GPU inside Docker | python3 -c "import torch; print(torch.cuda.is_available())" |
| Rebuild image after changes | sudo docker commit carbot_vision carbot:ready |
| Fresh container from saved image | sudo docker run ... carbot:ready bash |

Carbot System Guide · Jetson Orin NX